Teleport’s technical rider

The band needs not less than 40 minutes for soundcheck.

Equipment:
PA – average SPL at FOH place about  96dB A. Full access to all system’s controls. 
FOH console:

Not less than 16mic-line ins, 4-band EQ,  2 parametric bands for each channel.  Not less than 6 AUX

Analog consoles:  Midas, Allen&Heath (GL series), Soundcraft (GB series, LX7ii). 
FX: Lexicon, Tc-electronic (Hall, Reverb).
Dynamics: DBX, Drawmer, SPL at quantity according to input list.

Digital consoles:  Soundcraft (Si, Si+, Vi series), Behringer x32, Allen Heath (iLive series, Qu16, 32, Gld series).

Backline:
1. Drums - Yamaha, Pearl, Mapex, Sonor. 
Bass Drum - 22'' - 24'', toms 10’’-12’’, floor tom 14'' - 16'', 1 stand for snare drum, 1 for hihat, pedal. 
2. Bass - combo Ampeg, Hartke, SWR, active DI box, 220v power supply.

3. Guitar & VOX
Tube combo or stack (VOX, Orange, Fender) about 30W, clean&crunch channels. Marshall amps are strongly undesireable.

1 guitar stand.

1 synth stand for a small sampler (akai MPX16)
1 mic stand. 
Vocal mic (Shure sm58, beta 58). 
220V power (3)

Аux:
1st monitor line for guitaris/vocalist  (500W)

2nd  monitor line for bassist (500W)

3rd  monitor line for drummer (preferably with subwoofer)

5th AUX – for reverb.

6th AUX for delay. 
Input list:

| N | Instrument | source | (Insert) |
| --- | --- | --- | --- |
| 1 | BD Out | Shure Beta52, Beta92 | Gate/compressor |
| 2 | BD In | Shure Beta91 | Gate/compressor |
| 3 | SN Top | Shure Sm57, sennheiser 904 | Gate/compressor |
| 4 | SN Down | Shure Sm57 | Gate/compressor |
| 5 | HH | Shure sm81, any condenser small diaphragm mic (cardioid) |  |
| 6 | Rack Tom | Shure Beta56A, Shure Sm57 | Gate/compressor |
| 7 | Floor Tom | Shure Beta56A, Shure Sm57 | Gate/compressor |
| 8 | OH L | Shure sm81, any condenser small diaphragm mic (cardioid) |  |
| 9 | OH R | Shure sm81, any condenser small diaphragm mic (cardioid) |  |
| 10 | Bass Di | DiBox | comp |
| 11 | Guitar | Shure Sm57 | comp |
| 13 | Akai MPX16 sampler | DiBox (mono) |  |
| 15 | Voc Talash | Shure Sm58, beta58 | comp |

Stage Plot: