St Lucifer

VOCALS (ALEX)

LEAD VOCAL: 1 x microphone with stand – This has a Vocal Effects Unit (to go between the venue mic and multicore/desk)

MICROKORG: 1 x Mono DI

GUITAR/TECH/VOCALS (DAVID)

BACKING VOCAL 1: 1 x microphone and stand

GUITAR: Amp (Own) to be mic’d up

MAIN TECH: Stereo DI x – This includes all remaining synths/sounds and laptop through the stereo feed – It should be the first thing you line-check

BASS (JOHN)

BASS GUITAR: Amp (Own) to be DI’d via external DI or amp's built in DI

DRUMS/VOCALS (CHARLIE)

BACKING VOCAL2: 1 x microphone with stand

PERCUSSION: 1 x Mono DI for Percussion Output

Mixing & Set Up

Best way to view our set up is like a DJ set with additional vocals and instruments

The MAIN TECH the main thing to be heard out front and via the monitors and can be seen as ‘the backing track’. This should be as loud and as far forward in the mix as possible. This also contains all of the main percussion and sequencing

PERCUSSION, MICROKORG & GUITAR: All of these should not dominate the backing track but sit below it – The guitar is mostly adding wall of noise, while the percussion is essentially snares and hi-hats over the top of the MAIN TECH – The Microkorg is only adding very incidental parts on certain tracks – it should sit low in the mix

Bass Guitar:  The MAIN TECH has been pre-mixed to leave just the right amount of space for the Bass to sit within – You should treat the Bass as if it is a standard one playing along with a drum machine/electronic act

MAIN VOCALS should sit just above the backing track and not be too distinctive

BACKING VOCAL 1: DAVID Should be at the same level of the main vocals

BACKING VOCAL 2: CHARLIE Should be lower in the overall mix